Liquid Rocket Engine Physics

A short walkthrough of the quantities that set a small pressure-fed methalox chamber.

This note is written for the 25 lbf heat-sink article: gaseous oxygen and gaseous methane, bottle-fed, sea-level stand. The goal is not a textbook. It is the chain of decisions that turns “I want 25 pounds of thrust” into a throat diameter, a bottle pressure, and an oxidizer-to-fuel ratio that will not melt the wall.

Every equation below names its symbols on the spot. Repeating those names is intentional.

# What a rocket engine actually is

A liquid rocket is a controlled pressure vessel with a hole. Propellants are forced through an injector into a chamber, they burn, the resulting hot gas is choked at a throat and then expanded in a nozzle. Thrust is not “the fire.” Thrust is the momentum leaving the exit plus the pressure acting on the exit plane:

F=m ve+Pe−PaAe

where

- F is thrust (force; lbf or N),

- m is total propellant mass flow rate (mass/time; lbm/s or kg/s),

- ve is exhaust velocity at the nozzle exit (length/time; ft/s or m/s),

- Pe is static pressure of the gas at the nozzle exit plane,

- Pa is ambient (atmospheric) pressure at the test stand,

- Ae is nozzle exit area.

The first term is momentum thrust. The second is pressure thrust: positive when the nozzle is under-expanded (Pe>Pa), negative when over-expanded (Pe<Pa), zero when perfectly expanded (Pe=Pa).

Everything else in this primer exists to make m, ve, Pe, and Ae consistent with a feed system built from K-bottles.

# The two performance numbers that split the problem

Rocket performance is split into a chamber number and a nozzle number so chemistry is never mixed with geometry.

## Characteristic velocity, c*

c* (“c-star”) measures how well the chamber turns propellant mass into high-pressure gas before the nozzle does any work:

c*=Pc Atm

where

- c* is characteristic velocity (m/s or ft/s),

- Pc is chamber pressure, taken at the injector end (psia or Pa),

- At is throat area (in​2 or m​2),

- m is again the total propellant mass flow.

c* depends on flame temperature Tc, product molecular weight M, and the ratio of specific heats γ. Those come from chemistry (NASA CEA). A good injector and enough residence time approach the CEA value. A small heat-sink chamber leaks heat through the wall and usually lands at

ceff*=ηc* cideal*

where ηc* is combustion efficiency (dimensionless, typically 0.90–0.94 here) and cideal* is the CEA number.

## Thrust coefficient, CF

CF measures how well the nozzle turns chamber pressure into directed force:

F=Pc At CF  and  Isp=c* CFg0

where

- CF is the thrust coefficient (dimensionless),

- Isp is specific impulse (seconds) — thrust per unit weight-flow,

- g0=9.80665 m/s2=32.174 ft/s2 is standard gravity, used only to put Isp in seconds.

CF depends on γ, the expansion ratio ε=Ae/At, and the ambient-to-chamber pressure ratio Pa/Pc. It is almost independent of how hot the gas is.

That is why you pick oxidizer-to-fuel ratio and Pc first, read c* and γ from CEA, pick ε from the altitude you fire at, then Isp falls out. You never pick Isp and work backward.

# Pressure from bottle to chamber

On a pressure-fed engine the entire pressure budget is set by the bottles, not by “what would be a nice Pc.” Start at the tank and walk downstream.

| Station | Typical ΔP / P | Why it exists |
| --- | --- | --- |
| K-bottle, regulated | 500–600 psig usable | What the regulator can hold as the bottle blows down. |
| Lines, fittings, valves | small, but real | Keep short and large-bore. Every psi here is stolen from injector ΔP. |
| Injector drop, ΔPinj | ≈20% of Pc | Atomization plus stiffness. See next section. |
| Chamber, Pc | 250 psia (this article) | Set by feed headroom, not by peak c*. |
| Throat | ≈0.56 Pc (choked) | Sonic condition. Mass flow is locked once At and c* are known. |
| Nozzle exit, Pe | ≈Pa at design | Sea level: Pe≈14.7 psia. That fixes ε≈3 for γ≈1.2. |

For this engine: Pc=250 psia plus an injector drop of ΔPinj≈0.20 Pc≈50 psi, plus regulator margin, means the regulator must sit well above 300 psia. That is why 250 psia was chosen instead of 400 or 500 — higher Pc would give a slightly better c* and a smaller chamber, but the bottles would empty before the burn ended.

Here ΔP always means a pressure drop (upstream minus downstream) across one component.

# Injector ΔP, stiffness, and flow rate

The injector is a set of orifices. For incompressible liquid flow through an orifice,

m=Cd Ainj2ρ ΔPinj

where

- Cd is the discharge coefficient (dimensionless, typically 0.6–0.9 depending on orifice edge and L/D),

- Ainj is the total open area of the orifices for that propellant,

- ρ is propellant density at the injector,

- ΔPinj=Pman−Pc is the pressure drop from the injector manifold pressure Pman down to chamber pressure Pc.

Two consequences follow immediately.

#### Atomization.

ΔPinj becomes jet velocity

vinj≈2 ΔPinjρ

where vinj is injection velocity. Higher drop means a finer spray and better mixing. For gaseous methalox the “spray” is shear between the two gas streams, but you still need enough ΔPinj to set the velocity ratio and keep the elements from coupling to the chamber.

#### Stiffness.

Because m scales with ΔPinj, a large ΔPinj relative to Pc means chamber-pressure wobbles change the flow only a little. If Pc rises 10 psi and ΔPinj is 50 psi, flow barely moves. If ΔPinj is 10 psi, the same wobble dumps extra propellant into the chamber and you get chug — a low-frequency feed-coupled oscillation. The rule of thumb is

ΔPinjPc≈0.15–0.20 (sometimes up to 0.25)

That ratio is injector stiffness (or hardness). It is a stability requirement, not a performance target.

On a fixed-geometry injector, ΔPinj falls with the square of mass flow, so stiffness collapses when you throttle. This article is unthrottled: size the orifices for ∼20% at the design point and leave it.

Gaseous propellants are compressible, so the exact orifice equation is not the incompressible square-root form. The design idea is identical: pick a drop that isolates the feed system and sets injection velocity, then size Ainj to hit the required m.

# Mass flow, O/F, and why 2.8 not 3.3

Total mass flow is locked once Pc, At, and CF are known:

m=Pc Atceff*=FIsp g0

Mixture ratio splits that flow. Define the oxidizer-to-fuel ratio

r≡O/F=moxmfuel

Then

mox=m r1+r,  mfuel=m 11+r

where mox is oxidizer mass flow and mfuel is fuel mass flow.

CEA for GOX/GCH​4 at 250 psia puts peak theoretical c* near r≈3.3. Stoichiometric (complete combustion to CO​2 and H​2O) is r=4.0. Flight methalox engines usually sit around 3.4–3.6. This article uses r=2.8 — fuel-rich, about 1% off peak c*.

That 1% is the cheapest cooling you will ever buy. A heat-sink chamber has no regen channels. The wall survives the burn by thermal mass plus a fuel-rich boundary layer. Run peak-c* and the gas next to the copper (or steel) is hotter, more oxidizing, and the test is shorter. On a bottle-fed stand that trade is free on the performance side and expensive if you skip it.

Ideal c* from CEA at this point is about 1785 m/s. Apply ηc*=0.92 for a small heat-sink chamber (imperfect mixing + wall heat loss):

ceff*=0.92×1785 m/s≈1640 m/s.

The ratio of specific heats of the products is γ≈1.2 — not 1.4 like cold air. γ=cp/cv, where cp and cv are specific heats at constant pressure and volume. Use the CEA γ in every nozzle calculation.

# Expansion ratio is an altitude choice

A nozzle is correctly expanded when Pe=Pa. For sea level, Pa=14.7 psia and Pc=250 psia, so

PePc≈14.7250≈117.

For γ≈1.2, the isentropic area ratio that produces that pressure ratio is

ε≡AeAt≈3

where ε (epsilon) is the nozzle expansion ratio.

Larger ε over-expands: Pe drops below ambient, a shock sits in the bell, flow can separate, sea-level thrust falls. Smaller ε leaves pressure (and thrust) on the table. Altitude engines use ε of 20–80 because Pa is near zero. This article fires at the beach, so ε≈3 is the whole justification for the exit diameter.

Ideal CF at ε=3, γ≈1.2, sea level is about 1.38. A real short nozzle delivers

CF,eff=ηCF CF,ideal

with nozzle efficiency ηCF≈0.93 (divergence, boundary layer, incomplete recombination). That is how the basis line gets from ∼251 s ideal Isp down to ∼215 s delivered:

Isp,ideal≈1785 m/s×1.389.81 m/s2≈251 s,Isp,delivered≈0.92×1785 m/s 0.93×1.389.81 m/s2≈215 s.

# Geometry last, in this exact order

Once F, Pc, ceff*, and CF,eff are known, the throat is algebra:

At=FPc CF,eff

Then Ae=εAt. Chamber diameter comes from the contraction ratio Ac/At (typically 2–4 for a small chamber), where Ac is the chamber cross-section, and from how many injector elements you can pack on the face. Chamber length comes from

L*=VcAt

where L* (“L-star”) is characteristic chamber length and Vc is chamber volume up to the throat. L* is a residence-time number. Gaseous methalox does not need the long L* of a LOX/RP-1 engine, but a heat-sink article still wants enough length that the film and core can mix before the throat.

| # | You choose | Because |
| --- | --- | --- |
| 1 | F=25 lbf, Pc=250 psia | Stand scale + bottle headroom + injector stiffness. |
| 2 | r=O/F=2.8 | Fuel-rich film; heat-sink wall survival. |
| 3 | c*, γ from CEA | Chemistry. Then derate ηc*≈0.92. |
| 4 | ε≈3 | Sea-level matched expansion, Pe≈Pa. |
| 5 | Isp≈215 s | Falls out of c*CF/g0 after two efficiencies. |
| 6 | At, then Ae, then chamber | Algebra. Throat first. Everything else is a ratio. |

If you pick a 3-inch exit because it looks right and then back into ε, you have inverted the physics. The 8 mm throat implied by the original notes is this calculation run in the other direction: given At, back-solve cideal* and confirm it matches CEA for GOX/GCH​4 at r=2.8 and Pc=250 psia.

# Numbers you can put on a calculator

Keep Pc in consistent units with At — the most common error is mixing psia with mm​2 and expecting pounds.

| Quantity | Working value |
| --- | --- |
| Thrust F | 25 lbf (111 N) |
| Chamber pressure Pc | 250 psia (1.72 MPa) |
| O/F mass ratio r | 2.8 |
| cideal* (CEA) | ≈1785 m/s |
| ηc* | 0.92 → ceff*≈1640 m/s |
| γ (products) | ≈1.20 |
| ε=Ae/At | ≈3.0 (sea level) |
| CF,ideal | ≈1.38 |
| ηCF | 0.93 → CF,eff≈1.28 |
| Isp delivered | ≈215 s |
| ΔPinj/Pc | ≈0.20 → ΔPinj≈50 psi |
| Regulator / manifold Pman | ≥300 psia plus line loss |
| Bottle usable | ≈500–600 psig regulated K-bottle |
| g0 | 9.80665 m/s​2 or 32.174 ft/s​2 |

# What will actually bite you on the stand

- Heat, not thrust. A 25 lbf heat-sink chamber fails by wall temperature, not by missing 2 seconds of Isp. Size the burn from thermal mass and film coverage, then see how much bottle you have left — not the other way around.

- Chug on start. Ignition is the moment Pc is still climbing and ΔPinj/Pc is at its worst. Soft start, or accept that the first 100–200 ms is the stability-critical window.

- Mixture-ratio shift. Two independent gas legs with two regulators will not hold r perfectly as bottles blow down. Design the film to still protect the wall at the leanest r you will actually see.

- Units. c* in m/s, Pc in Pa, At in m​2 gives m in kg/s. Or use lbf⋅s/lbm, psia, and in​2 with g0=32.174. Pick one system and stay in it for a given calculation.

- Do not chase higher Pc. On bottles, pressure is a consumable. 250 psia is already a feed-system number. Raising it shrinks the engine and helps c* a little, then the burn ends early.

# One-page mental model

Bottles set the highest pressure you can spend. Spend ∼20% of chamber pressure across the injector so the feed system cannot hear the chamber. Spend the rest as Pc. Ask CEA what c* and γ that chemistry produces at the O/F that keeps the wall alive. Expand only as far as sea-level ambient allows. Multiply c* by CF, divide by g0, that is Isp. Divide thrust by Pc CF, that is the throat. Everything after the throat is a ratio.

That is the whole engine, in the order it should be designed.